#### ATTACHMENT – TECHNICAL SPECIFICATION FORM

TENDER No. 91189927 – 91189932

Bidders are requested to complete this form, sign it and return it as part of their bid submission. The bidder shall fill in this form in accordance with the instructions indicated. No alterations to its format shall be permitted and no substitutions shall be accepted.

This technical specification form has an integral part with the Bid Price Form for this tender.

Date: [Insert submission date]

Name of the Bidder: [Insert the company name]

Subject       [Insert the tender subject]                    , Inquiry  [tender no]    dated [insert date]

| No | Items and Requirement Specification | Confirmation of Compliance to the Specification | Proposed Brand Type & Specification (please mention the detail specification) | Picture (as sample, not mandatory brand) |
| --- | --- | --- | --- | --- |
| A | LOT 1 – 91189927, Compost Chopping Machine |  |  |  |
| 1 | Item 1: Compost Chopping Machine (1 unit) (Multifunction for Empty fruit bunches and palm fronds) |  |  |  |
|  | Minimum specification: - Equipped with 42 double-bladed steel knives - 30hp diesel engine - 2.8ml thick tube plate - 50cm tube diameter - Production capacity of 1 ton/hour - Spare parts are available in Indonesia - 1 year warranty |  | Note: The Bidder is not allowed to quote more than one item (options) with different price for one kind of item requested. The vendors who do not comply will be ruled out from the tender evaluation Catatan: Peserta tender tidak diijinkan untuk memberikan penawaran lebih dari satu jenis (pilihan) dengan harga yang berbeda untuk setiap item yang diminta. Vendor yang tidak mematuhi hal di atas, tidak akan diikut sertakan dalam evaluasi. Remarks: The specification proposed shall be clear, details (brand, type, details specs) and not ambiguous. This is very important part for the evaluation process. | The Bidder is obliged to attach the brochure of the items proposed |
| B | LOT 2 – 91189928, Fire extinguishers (Apar), Safety Helmet, First aid box |  |  |  |
| 2 | Item 1: Fire extinguishers (Apar) (20 units) |  |  |  |
|  | Minimum Specification: - Cylinder material: Steel St12 - Cylinder type: Stored-Pressure System - Media: ABC Powder - Capacity/Weight Full/Empty (kg): 6/8.5/2.5 - Height/Width/Diameter (cm): 55.5/24/14.5 - Discharge Range (m): 3 to 12 - Discharge Time (sec): 24 seconds - Test Pressure (bar): min 25 - Bracket on wall - Certified to SNI 180-1:2022 standards. - Warranty: 2 years |  |  |  |
| 3 | Item 2: Safety Helmet (200 pcs) |  |  |  |
|  | Minimum Specification: - Color: Mono/single color for 200 pcs (prefer: White) - SNI ISO 3873:2012 - Helmet Material: Polypropylene - Weight: 332.5 grams - Non-flammable |  |  |  |
| 4 | Item 3: First aid box (50 boxes) |  |  |  |
|  | Minimum Specification: Size: 30 x 11 x 29 cm Includes: Bandages and Gauze: - Sterile gauze pads (20 pieces). - Bandages (5 cm and 10 cm, 2 pieces each). - Adhesive tape (1.25 cm wide, 2 pieces). - Quick-adhesive tape (10 pieces). - Cotton wool (25 g). - Triangular bandage (mitella, 2 pieces). Tools: - Bandage scissors. - Safety pins (12 pieces). - Disposable gloves (2 pairs). - Masks (2 pieces). - Tweezers.1pcs - Mini Flashlight. 1pcs - Eye wash cup. Medications and fluids: - Clean plastic bags. - Aquades 100 ml (or YRINS 12 ml). - Povidone Iodine 60 ml. - 70% alcohol. Documents: - First aid manual. - Notebook. |  |  |  |
| C | LOT 3 – 91189929, Egrek Palm Knife + Cover |  |  |  |
| 5 | Item 1: Egrek Palm Knife + Sarung/Cover of Egrek knife (150 unit) |  |  |  |
|  | Minimum Specification: Blade (Egrek Blade) - Length: ± 45 cm - Blade width: ± 5–6 cm (widest part) - Thickness: ± 6 mm - Blade weight: ± 0.8–1 kg - Material: SK5 carbon steel Handle / Holder - Material: Lightweight iron pipe / aluminium alloy (rust-resistant, lightweight, strong) - Standard length: ± 2 m (can be extended with additional sections) - Outer diameter: ± 32–35 mm - Connection system: Socket/locking clamp Blade Protective Cover - Material: Thick synthetic leather / hard plastic (varies depending on distributor) - Length: ± 47–50 cm (adjusted to blade length) - Width: ± 7 cm - Material thickness: ± 3–5 mm |  |  |  |
| D | LOT 4 – 91189930, GPS Map |  |  |  |
| 6 | Item 1: GPS Map, include Memory Card (4 units) |  |  |  |
|  | Minimum Specifications: - DIMENSIONS: 2.67'' x 6.85'' x 1.67'' (6.77 x 17.40 x 4.23 cm) - DISPLAY SIZE: 1.5"W x 2.5"H (3.8 x 6.3 cm); 3" diag (7.6 cm) - DISPLAY RESOLUTION: 240 x 400 pixels - DISPLAY TYPE: transflective, 65K color TFT - WEIGHT: 9.95 oz (282g) with batteries - BATTERY TYPE: 2 AA batteries; NiMH or Lithium recommended - BATTERY LIFE: up to 19 hours - WATER RATING: IPX7 MIL-STD-810 yes (thermal, shock, water) - INTERFACE: high speed microUSB and NMEA 0183 compatible - MEMORY/HISTORY: 8 GB - WAYPOINTS/FAVORITES/LOCATIONS: 10,000 - TRACKS: 250 - TRACK LOG: 20,000 points, 250 saved gpx tracks, 300 saved fit activities - ROUTES: 250, 250 points per route; 50 points auto routing Sensors: - GPS - GLONASS - GALILEO - QZSS - COMPASS Yes (tilt-compensated 3-axis) - GPS COMPASS (WHILE MOVING) - POINT-TO-POINT NAVIGATION: YES - AREA CALCULATION: YES - HUNT/FISH CALENDAR: YES - GEOCACHING-FRIENDLY: Yes (Paperless) - CUSTOM MAPS COMPATIBLE: YES - PICTURE VIEWER YES - 1 year warranty Include: - USB data cable - Manual book - Memory Card 8 GB |  |  |  |
| E | LOT 5 – 91189931, Hand Traktor |  |  |  |
| 7 | Item 1: Hand Traktor (4 units) |  |  |  |
|  | Minimum Specifications: - Engine capacity: 198 cc - Cutting width: 75 cm - Cutting depth: 10 cm - Blade type: Dryland - Transmission gears: 2 forward - 1 reverse - Engine type: G 200 F - Engine RPM: 3600 rpm - Starting: Recoil (manual) - Fuel consumption/hour: 1.52 liters/hour - Fuel tank capacity: 3.6 liters - Oil capacity: 600 cc - Dimensions (LxWxH): 1500 x 850 x 1000 mm - Accessories: tool bag, bolts, blades, rubber tires, warranty card - Net weight: 63 kg - Unit and spare parts are available in Indonesia - 1-year warranty |  |  |  |
| F | LOT 6 – 91189932, High pressure water pump machine |  |  |  |
| 8 | Item 1: High pressure water pump machine (8 units) |  |  |  |
|  | Minimum Specifications: - Maximum Engine Power: 17 HP - Outlet and Inlet Pipe Diameter: 80 mm (3 inches) - Maximum Water Volume: 38 m³/hour - Maximum Spray: 55 m - Engine Speed: 3,600 rpm - Efficiency: 67% - Suction Depth: 7 m - Suction Time: 160 seconds - Product Weight: 50 kg - 1 year warranty |  |  |  |

| TERMS AND CONDITIONS: |  | Please tick “Yes” or ”No” |  | Deviations |
| --- | --- | --- | --- | --- |
| 1 | Product shall be delivered by the supplier after the PO release. | ☐ Yes ☐ No |  | Sample Not Applicable |
| 2 | The contractor shall deliver the goods after PO released and supplier signing the Purchase order. - Within 30 days, for LOT 1 (Compost Chopping Machine) - Within 2 weeks, for other items (LOT 2 – 6) Please state the number of days, if estimation delivery date will be beyond the required days mentioned above. Delivery address: Gudang GIZ Continental Ruko H. Jusman (Seberang Masjid Tua Mujan) Jalan Lintas Selatan, Desa Mujan, Kecamatan Boyan Tanjung, Kabupaten Kapuas Hulu, Kode Pos: 78758 Indonesia. PIC/recipient name will be informed at the end to the awarded contractor. | ☐ Yes ☐ No |  | … days |
| 3 | Warranty: - Minimum 1 year for electronic and machines - 2 years for Fire extinguishers | ☐ Yes ☐ No |  |  |
| 4 | Please to provide list of company experienced as distributor for agriculture equipment/tools or sample of PO to related items. |  |  |  |
| 5 | Validity of Quotation is 60 calendar days |  YES |  NO |  |
| 6 | Payment terms 30 days of receipt of the Supplier's invoice and delivery receipt of the goods specified in the Purchase Order. |  YES |  NO |  |
| 7 | The quality inspection shall be conducted when the goods delivered. Any defects shall be remedied by the Supplier without any delay. |  YES |  NO |  |
| 8 | Please provide After Sales Guarantee Letter signed over duty stamp (Materai) containing information about: if the tools condition upon arrival at project office are found damaged/ broken/ incomplete, the replacement will provided refer to the required specification (meet requirement) as mentioned in the technical specifications form. |  |  |  |
| 9 | The Supplier should be responsible for the (safety) packing and delivery to above address. |  YES |  NO |  |
| 10 | The offered item shall READY STOCK or please mentioned the availability information for each item(s) if indent. |  |  |  |

I, the undersigned, certify that I am duly authorized by [insert full name of bidder] to sign this quotation and bind [insert full name of bidder] should GIZ accept this quotation:

Name: [complete]

Title: [complete]

Date: [complete]

Signature and company stamp: _____________________________________________________________