#### ATTACHMENT – TECHNICAL SPECIFICATION FORM

TENDER No. 10051457

| No | Items and Requirement Specification | Confirmation of Compliance with the Specifications (YES or NO) If NO, please explain | Proposed Brand, Type & Specifications (Please mention the detailed specifications) | Picture (as sample, not mandatory brand) |
| --- | --- | --- | --- | --- |
| 1. | 2-unit of workstation (Dell/ Lenovo/HP/Asus/other brand) to perform AI model computations with specification: CPU GPU RAM Storage Power Supply Network Wireless Ports Operating System Software; NVIDIA GB10 Grace CPU (20-core ARM: 10 Cortex-X925 + 10 Cortex-A725) (equivalent or higher) NVIDIA GB10 Blackwell GPU (5th Gen Tensor, 4th Gen RT) 128 GB LPDDR5X Unified Memory (273 GB/s) 4TB M.2 2242 NVMe PCIe 5.0 240W Ethernet 1x RJ-45 10GbE, WiFi 7, NIC ConnectX-7 NIC @200Gbps or higher Wi-Fi 7 & Bluetooth 5.4 3× USB-C 20Gbps + 1× USB-C PD 1x HDMI 2.1a NVIDIA DGX OS 7 NVIDIA AI Software Stack. |  | Note: Suppliers are not allowed to submit multiple options with different prices for the same item requested. Any suppliers who do not comply with this requirement will be disqualified from the tender evaluation process. It is crucial that the specifications provided are clear, detailed (including brand, type, and specific details), and unambiguous. This is a very important part of the evaluation process. |  |
| 2. 3. | 1-unit of switch (Mikrotik/other brand), with specification: CPU; Quad-Core AL52400, 2.0 GHz (ARM 64-bit). RAM; 4 GB DDR4. Storage; 512 MB NAND. Ethernet Ports; 2 × 1G/2.5G/5G/10G Ethernet. High-Speed Ports; 4 × 400G QSFP56-DD. Switch Chip; 98DX7335. Operating System; RouterOS v7, License Level 6. Power Supply; Dual AC Input (100–240 V AC), 12 V / 27.5 A Power Adapter. Power Consumption; 92 W (without attachments), 123 W (maximum). 2-unit of cable, with specification: Model; QDD-2XQ56. Type; 400G QSFP-DD (8 × 50G PAM4) to 2 × 200G QSFP56 Passive Copper DAC Breakout Cable. Connector; 1 × QSFP-DD to 2 × QSFP56. Speed; 2 × 200G. Media; Passive Copper (Twinax DAC). Length; 2 meter. - Maintenance Service for 12 months - Warranty for 2 years |  |  |  |

| TERMS AND CONDITIONS: |  | Please tick “Yes” or ”No” |  | Deviations |
| --- | --- | --- | --- | --- |
| 1 | Delivery date after PO is release (please state number of days) or delivery plan/schedule Delivery Address: Directorate General of Taxes (DJP) Kantor Pusat DJP, Jalan Gatot Subroto Kav. 40-42, Jakarta, 12190 |  |  | ……………days Delivery plan |
| 2 | Items original with official warranty |  YES |  NO | Please state the warranty period |
| 3 | The items proposed are new and original. Please provide photos/pictures |  YES |  NO |  |
| 4 | Validity of Quotation is 30 calendar days |  YES |  NO |  |
| 5 | The supplier should be responsible for the packing |  YES |  NO |  |
| 6 | The quality inspection will be conducted upon delivery of the goods. Any defects will be promptly addressed by the supplier. |  YES |  NO |  |
| 7 | Availability of sending samples to GIZ (if needed) |  YES |  NO |  |
| 8 | Availability of spare-parts ex stock in Jakarta |  YES |  NO |  |
| 9 | Availability of after-sales/maintenance/repair service in Jakarta |  YES |  NO |  |
| 10 | Payment terms 2-3 months of receipt of the Supplier's invoice and delivery receipt of the goods specified in the Purchase Order |  YES |  NO |  |
| 11 | All prices/rates quoted must be exclusive of all taxes, since the GIZ, including its subsidiary organs, is exempt from taxes according to PMK No. 215/PMK 03/2008 and KMK No. 17/KMK 010/Year 2021. GIZ shall process the VAT with tax exemption to BADORA. The process of tax exemption will take 2 - 3 months. The supplier is obliged to release the Tax Invoice with code 08 (tax exemption). Please provide the PKP and/ PTKP Letter. |  YES |  NO |  |
| 12 | Supplier Self-Declaration to determine the origin or source of the goods offered shall be signed and attached with the tender document (obligatory) – as annex 3 (Harus di tandatangani dan di lampirkan pada saat submit penawaran) |  YES |  NO |  |
| 13 | Declaration of Prevention of Terrorism shall be signed and attached with the tender document – as annex 4 (Harus di tandatangani dan di lampirkan pada saat submit penawaran) |  YES |  NO |  |

I, the undersigned, certify that I am duly authorized by [supplier name] to sign this quotation and bind [supplier name] should GIZ accept this quotation:

Contact Person: name

Telp/Hp:

Job title:

Date:

Signature and company stamp: __________________